# MS747 MCP Router Quick User Guide

Version 1.2.13 | Windows | 29 September 2026

# Start a flight

1. Connect the panel's approved power adapter and USB cable. Extract the complete official router ZIP before running the app.

2. Open the router; install the USB driver or upload firmware only if prompted. Never unplug during firmware upload.

3. Select the correct simulator and Aircraft. Asobo and PMDG are different profiles. For PSX, set Host / Port and click Apply.

4. Load and power the aircraft cockpit. Confirm MCP Status and Simulator Status, then use Manual Sync. Leave the router running while flying.

# Asobo 747 setup after an update

Completely close MSFS. Open Debug & Options > MSFS WASM Bridge, confirm the installation, and click Install / Update WASM Bridge. Start MSFS again, load Asobo 747, check live status, and use Manual Sync. Installed files alone do not confirm a live connection.

# Adjust knobs and reversed directions

Encoder Mapper: choose a knob, set Maximum Acceleration (PMDG / Asobo, x) from 1 to 8, and Save Encoder. Slow turns stay fine; faster turns gain speed. Select 1 for no acceleration. PSX uses its separate sensitivity setting.

Direction Reverse: check only the affected F/D L/R, A/T ARM, DISENGAGE, IAS, HDG, ALT, or V/S item; click Save and retest normal operation. Settings persist. Hardware Test raw counts are not reversed.

# Check the hardware

Use HARDWARE TEST while not flying. Push each encoder switch three times. Turn every knob repeatedly both ways and watch the counts. Inspect lights and digits before Confirm. Skip is not a pass. Close the test and use Manual Sync.

# Troubleshooting

Panel not detected: check power/cable, try another USB port, avoid unreliable hubs, close other panel apps, then reconnect and restart the router.

Simulator not connected: select the matching profile and load the cockpit. Check PSX Host/Port, PMDG setup prompts, or Asobo support-package live status.

Numbers differ or knob is too slow: check profile, aircraft power and display brightness; use Manual Sync. Adjust acceleration and save. If values keep changing after you stop, create a support report.

A light stays off: compare the cockpit light, check power/brightness/color, then Hardware Test. Asobo's aircraft lamp test checks illumination, not normal mode engagement; several normal modes remain unverified in this release.

# Updates background use and support

Export settings before a manual reinstall. The built-in updater preserves settings. Hide to Background keeps the router in the tray; ordinary minimize does not hide it.

Debug & Options > Report a Problem: describe the issue, generate the report, review it, and attach the ZIP when submitting on GitHub or using Email Instead. Nothing is sent automatically.

Full guide and download: www.mikesierraaero.com/general-1 | Support: msaerodev@gmail.com. PC flight simulation only; no native Mac router.